### American International University- Bangladesh

Department of Electrical and Electronic Engineering

EEE4103: Microprocessor and Embedded Systems Laboratory

Title:  Implementation of traffic runway lights using timer functions.

Introduction: The objective of this experiment is to get familiarized with Timers and use them for the implementation of a traffic control system.

Theory and Methodology:

Timer: Every electronic component of a sequential logic circuit works on a time base. This time base helps to keep all the work synchronized. Without a time, base, devices would have no idea as to when to perform particular actions. Thus, the timer is an important concept in the field of electronics.

A timer/counter is a piece of hardware built into the Arduino controller. It is like a clock and can be used to measure time events.  A timer is a register whose value increases/decreases automatically.

In AVR, timers are of two types: 8-bit and 16-bit timers. In an 8-bit timer, the register used is 8-bit wide whereas, in a 16-bit timer, the register width is 16 bits.  This means that the 8-bit timer is capable of counting 28=256 steps from 0 to 255. Similarly, a 16-bit timer is capable of counting 216=65536 steps from 0 to 65535.

Experimental setup:

Apparatus:

- Arduino Uno/ Arduino Mega

- LED lights (YELLOW, RED, and GREEN)

- Resistors (220 ohms)

Code implementation of a traffic system with Timer:

#define RED_PIN 8 //define name of pins used #define YELLOW_PIN 10 #define GREEN_PIN 12 //define the delays for each traffic light color int red_on = 3000; //3s delay int red_yellow_on = 1000; //1s delay int green_on = 3000; //3s delay int green_blink = 500; //.5s delay int yellow_on = 1000; //1s delay int delay_timer (int milliseconds) { int count = 0; while(1) { if(TCNT0 >= 16) // Checking if 1 millisecond has passed { TCNT0=0; count++; if (count == milliseconds) //checking if required milliseconds delay has passed { count=0; break; // exits the loop } } } return 0; } void setup() { //define pins connected to LEDs as outputs pinMode(RED_PIN, OUTPUT); pinMode(YELLOW_PIN, OUTPUT); pinMode(GREEN_PIN, OUTPUT); //set up timer TCCR0A = 0b00000000; TCCR0B = 0b00000101; //setting pre-scaler for timer clock TCNT0=0; } void loop() { //to make red LED on digitalWrite(RED_PIN, HIGH); delay_timer(red_on); //to turn yellow LED on digitalWrite(YELLOW_PIN, HIGH); delay_timer(red_yellow_on); //turning off RED_PIN and YELLOW_PIN, and turning on greenLED digitalWrite(RED_PIN, LOW); digitalWrite(YELLOW_PIN, LOW); digitalWrite(GREEN_PIN, HIGH); delay_timer(green_on); digitalWrite(GREEN_PIN, LOW); //for turning green Led on and off for 3 times for(int i = 0; i < 3; i = i+1) { delay_timer(green_blink); digitalWrite(GREEN_PIN, HIGH); delay_timer(green_blink); digitalWrite(GREEN_PIN, LOW); } //for turning on yellow LED digitalWrite(YELLOW_PIN, HIGH); delay_timer(yellow_on); digitalWrite(YELLOW_PIN, LOW); }

Questions for report writing:

- Include all codes and scripts into the lab report following the writing template mentioned in appendix A of Laboratory Sheet Experiment 3.

- Implement this system using an online simulation platform www.tinkercad.com/www.wokwi.com.

- Configure the system to have delays for outputs according to your ID. Consider the last three digits from your ID (if your ID is XX-XXABC-X then consider A for the RED light, B for the YELLOW LED, and C for GREEN LED). Include the program and results within your lab report.

Reference(s):

1) https://www.arduino.cc/.

2) ATMega328 manual

3) https://www.avrfreaks.net/forum/tut-c-newbies-guide-avr-timers

4) http://maxembedded.com/2011/06/avr-timers-timer0/