- Develop an assembly program for an Arduino UNO to set the Sleep Mode Control Register (SMCR) to enter the power-down mode while keeping only Timer0 and Timer1 active using power reduction register (PRR).

Ans:

.include “avr/io.h”

.include “avr/sleep.h”

.equ SMCR = 0x53

.equ PRR = 0x64

LDI R16, 0b00000101

OUT SMCR, R16

LDI R15, 0b11010111

OUT PRR, R15

rjmp

2. Determine the output of the following program. Compute the duration of Arduino blinks an LED and makes the system power down. When powered down, the Arduino’s current drops from 10 mA to just 8 mA. The supply voltage is 3.3 V. Compute the amount of power that is saved by the Arduino during this power-down mode.​

​

#include "LowPower.h"​

​

void setup() {​

pinMode(10, OUTPUT); }​

​

void loop() {​

digitalWrite(10, HIGH);​

delay(1000);​

digitalWrite(10, LOW);​

LowPower.powerDown(SLEEP_4S, WDT_OFF, BOD_OFF, SPI_OFF, TWI_OFF);​

}​

​

Solution:​

If the above program is run, then the time for which the LED is ON is 1000 ms or 1 s because the digitalWrite() function sends a ‘HIGH’ signal to pin 10, where an LED is connected, and then the delay() function makes a delay of 1000 ms. After that, the digitalWrite() function sends a ‘LOW’ signal to pin 10. After that, there is no delay()function, but the device goes into power-down mode for 4 s. As such, the duration that the Arduino blinks an LED connected to pin 10 is every 5 s, that is, the LED is ON for 1 s and OFF for 4 s, and this loop continues.​

​

Initially, when the power is not down then the device draws a 10 mA current. Hence, it consumes a power of 3.3×10 = 33 mW.​

When the device is in low-power power-down mode, then the device draws an 8 mA current. Hence, it consumes a power of 3.3×8 = 26.4 mW.​

So, the amount of power that is saved by the Arduino during the low-power power-down mode = 33 – 26.4 = 6.6 mW.​