1. A space research facility is planning to shoot high powered high frequency pulses into space for research purposes. These pulses are to be shot at a 2 µs interval. The facility has an ATMega328P MCU to count the time. Every 2 µs, a timer interrupt will be triggered by the MCU and the pulse will be shot. For this purpose, it was decided that the Timer0 interrupt will be used. If the MCU is running at 16 MHz, prepare a program that triggers Timer0 interrupt every 2 µs. Timer0 is an 8-bit timer, and the available pre-scalers are 1, 8, 64, 256, and 1024.

[Hint: Bits 0 to 2 of the TCCR0B register must be set for the required pre-scaler value to be used. Output Compare Register OCR0A holds the necessary Timer0 count to achieve the required delay. Bit 1 of TIMSK0 is OCIE0A that must be set to enable comparison match with OCR0A. Any bits not mentioned here are to be considered as 0. All registers mentioned are 8-bit registers.]

Ans: Same as the code given in the Lecture-6a slide.

2. Determine the output of the following program and make corrections, if needed:

volatile boolean var_b = false; void isr_f() { var_b = fals; DigitalWrite(7, High); } void setup () { atachinterrupt (digitalPinToInterrupt2, isr_f, CHANGE); Pinmode(7, OUTPUT); digitalwrite(7, LOW); } void loop () { if (var_b) { // interrupt has occurred} } NB. Consider an LED is connected to pin 7 and a switch is connected to pin 2

Ans:

volatile boolean var_b = false;  // Initialize var_b

void isr_f() {

var_b = false;  // Corrected spelling of 'false'

digitalWrite(7, HIGH);  // Corrected 'High' to 'HIGH' & ‘Digital’ to ‘digital’

}

void setup() {

attachInterrupt(digitalPinToInterrupt(2), isr_f, CHANGE);  // Corrected attachInterrupt syntax and 2 to (2)

pinMode(7, OUTPUT);  // Corrected 'Pinmode' to 'pinMode'

digitalWrite(7, LOW);  // Corrected 'digitalwrite' to 'digitalWrite'

}

void loop() {

if (var_b) {

// Interrupt has occurred, but var_b is set to false inside ISR

// This block will never execute unless 'var_b' is set to true somewhere

}

}


The output of the program: 

> Initially, var_b is false, and pin 7 is set to LOW.

> When an interrupt occurs on pin 2, isr_f() is called.

> Inside the ISR, var_b remains false and digitalWrite(7, HIGH) sets pin 7 to HIGH.

>  Since var_b is never set to true, the if (var_b) condition in loop() will never execute.

3. Determine the output of the following program:

volatile boolean var_b = false;

void isr_f() {

var_b = true;

digitalWrite(3, HIGH);

}

void setup() {

attachInterrupt(digitalPinToInterrupt(5), isr_f, CHANGE);

pinMode(3, OUTPUT);

digitalWrite(3, LOW);

}

void loop() {

if (var_b) {

delay(1000);

digitalWrite(3, LOW);

var_b = false;

}

}

4. volatile boolean Output_Signal = true;

void isr() {

Output_Signal = !Output_Signal;

digitalWrite(8, Output_Signal);

}

void setup () {

pinMode(8, OUTPUT);

attachInterrupt (digitalPinToInterrupt (2), isr, RISING);

EICRA = 0b00000011;

sei();

EIMSK = 0b00000001;

}

void loop () {

digitalWrite(8, Output_Signal);

}